# Lab 5: Electrochemistry, Potentiometric Titrations, and Solubility Equilibria

# Data Analysis

Part A

- Create a plot of Ecell versus log 1Ag+ . Include your line of best fit and R2 on the plot. Properly format the plot and include a descriptive figure caption below your figure. Explicitly write out how the line of best fit equation relates to the Nernst equation and your experimental Nernst equation.

- Determine the slope from the linear fit and report it with the appropriate 95% CI. Use appropriate significant figures and don’t forget units.

- What is the theoretical number of electrons required in this half reaction at the indicator electrode? Calculate the experimentally determined number of electrons in this half reaction. Show your work.

- Determine E0cell from the linear fit and report it with the appropriate 95% CI. Use appropriate significant figures and don’t forget units.

- Calculate E0Ag/Ag+ from your E0cell value. Be sure to use propagation of error to calculate the appropriate 95% CI. Compare your E0Ag/Ag+ value to the literature value for the standard reduction potential of silver from the CRC Handbook value using a percent error. Show your work.

Part B

- Using your data from the careful titration in part B of 150 ppm NaCl, create an overlaid graph of voltage versus volume (titration curve) and the first derivative plot. Be sure to label all axes, the two curves a legend, and the equivalence point: (volume, voltage). Include a descriptive figure caption below your figure.

- Using the data from at least five groups of students of the careful titration of 150 ppm NaCl, calculate the average cell voltage at the equivalence volume and the 95% CI. Use appropriate significant figures and don’t forget units. Show your work. (Note: Be sure to include your own cell voltage at the equivalence point from your answer to Q6.)

- Calculate the average Ksp of AgCl and the 95% CI  using the from the five groups of students for the 150 ppm NaCl titration. Show your work and use appropriate significant figures.

- Calculate the percent error between the experimental Ksp of AgCl calculated from the 150 ppm NaCl titration and the theoretical value of the Ksp of AgCl (1.8 x 10-10). Show your work.

Part C

- Using your data from the careful titration in part C of unknown [NaCl], create an overlaid graph of voltage versus volume (titration curve) and the first derivative plot. Be sure to label all axes, the two curves a legend, and the equivalence point: (volume, voltage). Include a descriptive figure caption below your figure.

- Using the data from at least five groups of students of the careful titration of unknown [NaCl], calculate the average cell voltage at the equivalence volume and the 95% CI. Show your work.  (Note: Be sure to include your own cell voltage at the equivalence point from your answer to Q11.)

- Calculate the average Ksp of AgCl and the 95% CI  using the data from the five groups for unknown NaCl titration. Show your work and use appropriate significant figures.

- Calculate the percent error between the experimental Ksp of AgCl calculated from the unknown NaCl titration and the theoretical value of the Ksp of AgCl (1.8 x 10-10). Show your work.

- Calculate the unknown [NaCl] and 95% CI using the volume of AgNO3 (aq) titrated to reach the equivalence point for the five groups. Please use ppm as the unit. Show your work and use appropriate significant figures. (Remember that the Nernst equation uses concentrations in M and you will have to convert the final answer into ppm.)

# Discussion

- Why does the concentration of Cl- (and thus Ag+) in the cell cause changes in the voltage between the two electrodes? Describe what happens before, during, and after the equivalence point to help describe why the titration causes changes in the voltage.

# Instrumentation

- In general, what happens at the cathode? What happens at the anode?

- What is the purpose of a reference electrode?

- What is happening at the Cu/CuSO4 reference electrode? Write the reaction and describe what is happening.